# Generated Forum Workbook

Question 1 of 1

#### Activity title: Securing communication using Site-to-Site IPsec VPN

#### Scenario: Securing Communication Between ADIB Bank's Sharjah and Abu Dhabi Branches

#### ADIB Bank operates two key branches in Sharjah and Abu Dhabi. To protect the transmission of sensitive financial data between these branches, it's crucial to implement a secure communication channel. As the appointed Network Security Engineer, your responsibility is to establish a secure Site-to-Site IPsec VPN over the public internet, ensuring encrypted and safe data exchange between both locations. This VPN will safeguard the bank’s data integrity and confidentiality, enabling the seamless and secure transfer of information between the two branches.

#### Network Overview:

- Sharjah Branch:

- LAN Network: 192.168.1.0/24

- Router LAN Interface: 192.168.1.1

- Router WAN Interface: 10.1.1.2

- WAN Network: 10.1.1.0/30

- Abu Dhabi Branch:

- LAN Network: 192.168.2.0/24

- Router LAN Interface: 192.168.2.1

- Router WAN Interface: 10.2.2.2

- WAN Network: 10.2.2.0/30

#### Task 1:

#### Create Network Topology using packet tracer

#### Task 2: Configure IP Addresses on both PC’s:

#### For PC0 do the below configuration:

#### static ip address as 192.168.1.3

#### subnet mask-255.255.255.0

#### default gateway-192.168.1.1

#### For PC1 do the below configuration:

#### static ip address as 192.168.2.3

#### subnet mask-255.255.255.0

#### default gateway-192.168.2.1

#### Task 3: Configuring IP Addresses on all the Routers

#### Sharjah Branch Router (Router 0)

#### Configure the LAN and WAN interfaces:

#### R0>enable

#### R0# configure terminal

#### R0(config)# interface GigabitEthernet 0/0

#### R0(config-if)# ip address 192.168.1.1 255.255.255.0

#### R0(config-if)# no shutdown

#### R0(config-if)# exit

#### R0(config)# interface s0/0/0

#### R0(config-if)# ip address 10.1.1.2 255.255.255.252

#### R0(config-if)# no shutdown

#### Abu Dhabi Branch Router (ADIB-AUH-R1)

#### Configure the LAN and WAN interfaces:

#### R1>enable

#### R1# configure terminal

#### R1(config)# interface GigabitEthernet 0/0

#### R1(config-if)# ip address 192.168.2.1 255.255.255.0

#### R1(config-if)# no shutdown

#### R1(config-if)# exit

#### R1(config)# interface s0/0/0

#### R1(config-if)# ip address 10.2.2.2 255.255.255.252

#### R1(config-if)# no shutdown

#### Intermediate Router (R2)

#### Configure WAN interfaces between Sharjah and Abu Dhabi:

#### R2>enable

#### R2# configure terminal

#### R2(config)# interface s0/0/0

#### R2(config-if)# ip address 10.1.1.1 255.255.255.252

#### R2(config-if)# no shutdown

#### R2(config-if)# exit

#### R2(config)# interface s0/0/1

#### R2(config-if)# ip address 10.2.2.1 255.255.255.252

#### R2(config-if)# no shutdown

#### Task 4: configure static routes on all Routers

#### Sharjah Router (R0)

- Add a static route to reach the Abu Dhabi LAN (192.168.2.0/24) via the intermediate router’s IP (10.1.1.1):

#### R0(config)# ip route 192.168.2.0 255.255.255.0 10.1.1.1

- Now Add a static route to reach (10.2.2.0/30) via the intermediate router’s IP (10.1.1.1)

#### R0(config)# ip route 10.2.2.0 255.255.255.252 10.1.1.1

#### Abu Dhabi Router (R1)

#### 1. Add a static route to reach the Sharjah LAN (192.168.1.0/24) via the intermediate router’s IP (10.2.2.1):

#### R1(config)# ip route 192.168.1.0 255.255.255.0 10.2.2.1

#### 2. Now Add a static route to reach (10.1.1.0/30) via the intermediate router’s IP (10.2.2.1)

#### R1(config)# ip route 10.1.1.0 255.255.255.252 10.2.2.1

#### Intermediate router(R2):

#### Add static routes to forward traffic between Sharjah and Abu Dhabi networks:

#### R2(config)# ip route 192.168.1.0 255.255.255.0 10.1.1.2

#### R2(config)# ip route 192.168.2.0 255.255.255.0 10.2.2.2

#### Task 5: Testing connectivity between two LAN’s

#### Ping PC1 from PC0 and vice versa to make sure the connectivity

#### Task 6: Configure IPsec Parameters on R0

#### Step 1: Enable the Security Technology package.

#### a. On R0, issue the show version command to view the Security Technology package license information.

#### b. If the Security Technology package has not been enabled, use the following command to enable the package.

#### R0(config)# license boot module c2900 technology-package securityk9

#### c. Accept the end-user license agreement.

#### d. Save the running-config and reload the router to enable the security license.

#### e. Verify that the Security Technology package has been enabled by using the show version command.

#### Step 3: Identify interesting traffic on R0.

#### Configure ACL 110 on R1 to the LAN on R2 as interesting.

#### R0(config)# access-list 110 permit ip 192.168.1.0 0.0.0.255 192.168.2.0 0.0.0.255

#### Step 4: Configure the IKE Phase 1 ISAKMP policy on R0.

#### R0(config)# crypto isakmp policy 10

#### R0(config-isakmp)# encryption aes 256

#### R0(config-isakmp)# authentication pre-share

#### R0(config-isakmp)# group 5

#### R0(config-isakmp)# exit

#### R0(config)# crypto isakmp key secret address 10.2.2.2

#### Step 5: Configure the IKE Phase 2 IPsec policy on R0.

#### create the transform-set VPN-SET to use esp-aes and esp-sha-hmac.

#### R0(config)# crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

#### b. Create the crypto map VPN-MAP that binds all of the Phase 2 parameters together. Use sequence number 10 and identify it as an ipsec-isakmp map.

#### R0(config)# crypto map VPN-MAP 10 ipsec-isakmp

#### R0(config-crypto-map)# description VPN connection to R1

#### R0(config-crypto-map)# set peer 10.2.2.2

#### R0(config-crypto-map)# set transform-set VPN-SET

#### R0(config-crypto-map)# match address 110

#### R0(config-crypto-map)# exit

#### Step 6: Configure the crypto map on the outgoing interface.

#### Bind the VPN-MAP crypto map to the outgoing Serial 0/0/0 interface.

#### R0(config)# interface s0/0/0

#### R0(config-if)# crypto map VPN-MAP

#### Task 7: Configure IPsec Parameters on R1

#### Step 1: Enable the Security Technology package.

#### a. On R1, issue the show version command to verify that the Security Technology package license information has been enabled.

#### b. If the Security Technology package has not been enabled, enable the package and reload R1.

#### Step 2: Configure router R1 to support a site-to-site VPN with R1.

#### Configure reciprocating parameters on R1. Configure ACL 110 identifying the traffic from the LAN on R1 to the LAN on R0 as interesting.

#### R1(config)# access-list 110 permit ip 192.168.2.0 0.0.0.255 192.168.1.0 0.0.0.255

#### Step 3: Configure the IKE Phase 1 ISAKMP properties on R1.

#### Configure the crypto ISAKMP policy 10 properties on R1 along with the shared crypto key secret.

#### R1(config)# crypto isakmp policy 10

#### R1(config-isakmp)# encryption aes 256

#### R1(config-isakmp)# authentication pre-share

#### R1(config-isakmp)# group 5

#### R1(config-isakmp)# exit

#### R1(config)# crypto isakmp key secret address 10.1.1.2

#### Step 4: Configure the IKE Phase 2 IPsec policy on R1.

#### a. Create the transform-set VPN-SET to use esp-aes and esp-sha-hmac.

#### R1(config)# crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

#### b. Create the crypto map VPN-MAP that binds all of the Phase 2 parameters together. Use sequence number 10 and identify it as an ipsec-isakmp map.

#### R1(config)# crypto map VPN-MAP 10 ipsec-isakmp

#### R1(config-crypto-map)# description VPN connection to R0

#### R1(config-crypto-map)# set peer 10.1.1.2

#### R1(config-crypto-map)# set transform-set VPN-SET

#### R1(config-crypto-map)# match address 110

#### R1(config-crypto-map)# exit

#### Step 5: Configure the crypto map on the outgoing interface.

#### Bind the VPN-MAP crypto map to the outgoing Serial 0/0/1 interface. Note: This is not graded.

#### R1(config)# interface s0/0/0

#### R1(config-if)# crypto map VPN-MAP

#### Task 8: Verify the IPsec VPN

#### Step 1: Verify the tunnel prior to interesting traffic.

#### Issue the show crypto ipsec sa command on R0. Notice that the number of packets encapsulated, encrypted, decapsulated, and decrypted are all set to 0.

#### Step 2: Create interesting traffic.

#### Ping PC-0 from PC-1.

#### Step 3: Verify the tunnel after interesting traffic.